# Channel Editor For Twin Tuner Models.

# Ryu, Kiseon(ksryu@handan.co.kr). Handan broadinfocomm. Co. Ltd.

- Database Up/Download

<figure 1> Database Upload Dialog

You can obtain STB database just by clicking upload button() on the tool bar. <Transfer->Upload> menu has an same effect. Click the file button on the upload dialog and select the filename to save database. Database uploading takes a long time (3~5 min.) compared to downloading (~1 min.). After editing the database, you can download edited thing by choosing download button.

- Database Open/Close/Save

You can open/close/save the database by the icons below.

- Edit System Information

<figure 2> Edit System Information

There is a way to edit system parameters such as Language, User Profile, Parental Lock, TV system on the <System> tab. As soon as clicking the menu to change, you can show the list to choose. But, you need a password to change parental control option.

- Edit Satellite & TP Information

<figure 3> Edit Satellite Information

On the <Satellite> tab, you can do the job related to satellite and transponder such as TP add/delete, Chanel add/delete, Edit TP Parameter. Each menu will be shown by right-button clicking the items except edit TP(double clicking TP item).

<figure 4> Edit TP Dialog

- Edit Channels

<figure 5> Edit Channels

In this tab, you can do following jobs.

- To view the channel list by tuner, media(tv or radio), favorites, networks, CAS system. Each selection can be mixed up. (ex. Viewing channels belong to fav1 and viaccess)

- Add or Delete for each selection. You can add the channels to fav1 if you press ( <- ) button after selecting the channels in All Channels List. In addition, you can add the channels to each selection at a time. (For example, Adding the fav1 and conax attribute to the channels at the same time). Removing the attributes from the channels is very similar to the upper except using ( -> ) button after selecting the channels in Selected Channels List.

- Viewing channel informations. It is possible that you see the channel information. Just click the channel item. The channel information will be shown in bottom-left channel info. box. Red icon before channel name means this channel is scrambled and Blue icon is not.

- Editing Channel Information. The edit channel dialog will be appeared after you double-click the channel item in All Channel List. Following tasks are possible. Changing channel name, Channel locking, Adjusting volume offset, Changing PIDs. Such jobs are also possible on the right-button menu except channel name change, PID change.

- FAV and Channel Move. After you select the channels to move, drag and drop the selection to the position to locate. In All Channel List, It can effect channel move. If you do same thing in Selected Channel List under only fav selection state, FAV move is possible.

<figure 6> Edit Channel Dialog